ARKANSAS PACKET NETWORK NODEOP GUIDE

This is a list  with explanations of  the X1J4 commands that

are used by node operators or SYSOPs  in the Arkansas Packet

Network (APN). There are  more  commands, but most are never

used and are not included here, so as to simplify the guide.

This maunal IS NOT intended for USERS! A separate manual can

be made available with those commands that are accessible by

users, and containing more detailed explanations.

ALIAS

The ALIAS command allows the node's alias to be changed. The

syntax is :

ALIAS [ * | new-alias ]

If  no  parameter is given, or if the user is not  SYSOP  or

MANAGER,  the  current alias is displayed. If the  alias  is

deemed  to be a valid alias, the node's alias is changed  to

the new one entered. If the sysop gives the parameter of '*',

the  node's alias is cleared.

BBS

The syntax of the command is :

BBS [ * | ? | callsign ]

With  no  parameter,  the  command  connects  to  a  station

previously   specified  by  the  sysop.  Setting   the   BBS

destination  is  done by the use of the BBS command  with  a

callsign  as  a second parameter. Setting the BBS  to  allow

this  may only be done by a sysop. The '*' option  may  also

only  be  executed  by  the sysop,  this  command  clears  a

previously specified BBS.

The '?' option ( or any text if not sysop ), prints out  the

current BBS station setting.

If no BBS is set, the command issues an error message if  it

is invoked with no other parameters.

The  idea  of  this command is that,  like  with  the  'BBS'

command  of  the 'BPQ software, a user may  connect  to  the

local BBS from the node.

1

BTEXT

The  BTEXT  command sets or displays the  additional  beacon

text sent along with the beacon packets.

The syntax of the command is :

BTEXT [ * | message ]

With no parameter, the current message is displayed. If  the

user is also a sysop, and if text follows the command,  that

text  is  added to the current beacon text. If  the  message

starts  with  a  '*', the beacon text  message  is  deleted.

Hence,  to  clear the message, type the command  'btext  *'.

BYE or QUIT

There  are  no parameters to these commands.  When  entered,

they terminate the session. Both commands do the same thing.

CALIBRATE

This  command  allows  remote  calibration  checks  of   the

transmitter deviation. Its syntax is

CALIBRATE period [ toggle ]

The  period ( 1 to 60 seconds ), is the time for  which  the

transmitter  will  key  up for with  constant  tone.  It  is

undefined  as  to  which tone will be sent.  If  the  second

parameter  is given, the node will toggle between the  tones

every  [toggle]  seconds. The toggle must be between  1  and

[period] seconds. If a period is not given, the user is  not

sysop  or manager, or if it is out of range, the command  is

ignored. Note - quite  often it can appear that the node has

locked  up  having  failed to transmit  the  full  calibrate

period.  In fact, this is usually the hardware PTT  watchdog

in the TNC.  The node  thinks it is  still  sending  but the

hardware timer has removed the PTT signal.

CONNECT

This command allows  a station to connect to another station

or node. That node does not have to be a neighbor node,  but

may be any other node in the network provided that node does

show up when the NODE command is displayed.

CONNECT [callsign / alias]

2

CQ

This command allows a station to broadcast a CQ message from

the node.

CQ [message]

CTEXT

The CTEXT command sets or displays a message sent to a  user

who connects to the node by uplinking to the node's alias.

The syntax of the command is :

CTEXT [ * | message ]

With no parameter, the current message is displayed. If  the

user is also a sysop, and if text follows the command,  that

text  is added to the current connect text. If  the  message

starts  with  a '*', the connect text  message  is  deleted.

Hence,  to  clear the message, type the command  'ctext  *'.

A  message is only sent if there is a ctext message set  and

if the relevant bit is set in the mode command parameter.

HELP

This command displays text that was burned into the chip that

gives users a rudimentary explanation of user level commands.

HELP

HOST

The syntax of the command is :

HOST [ * | ? | callsign ]

This command is very similar to the 'BBS' command. It allows

connection  to  a  local  host, BBS  or  other  server.  The

difference  however,  is that as long as the TNC is  not  in

crosslinked,  and  if  a  callsign  is  not  set, the 'host'

command connects to the local RS-232 port.

The  idea  of  this command is that,  like  with  the  'BBS'

command  of  the 'BPQ software, a user may  connect  to  the

local  BBS,  another  node or server  from  this  node.  For

example,  a local community mailbox.

3

INFO

This command displays an information message to the user.

INFO [ * | info text ]

If a sysop enters the command followed by a *, it will erase

any previously sysop entered message.  It  cannot  erase any

message that was burned into the chip.  If the sysop  enters

the command followed by text,  that text  will be displayed.

If there is text  permanently  burned into the chip, it will

displayed after the permanent text.

LINKS

This  command shows the current level 2 links to  the  node.

Displayed one per line, the two callsigns are shown followed

by the link state, port number and current retry count.

MHEARD

The  TNC can be instructed to keep a list of the  last  'nn'

stations heard, where 'nn' is an integer between 1 and  100.

It can also be disabled. The syntax of the command is :

MHEARD [ nn ]

The  parameter is optional and only operates for the  sysop.

It  sets  the maximum length of the list.  Setting  to  zero

disables the function. The  heard list uses free buffers for

the list,  so a large  setting  means less  RAM for the node

software.

The  list  is  maintained  as linked  list,  with  the  most

recently  heard station first. The display shows the  number

of packets heard from that station and the time since it was

last  heard, in hours minutes and seconds.

Every  hour the list is checked for stations that  have  not

been  heard for 12 hours, and any such stations are  removed

from the list.

4

MODE

The following parameters may be configured :

1  The host mode

0 - RS-232 port standard.  1 - connected to device

2  The CWID send period

0 to 3600   0 disables it

3  The CWID keyer speed

4 to 10   30 to 12 WPM respectively

4  The port nodes broadcast control

0 - none  1 - radio port  2- RS-232   3 - both ports

5  The crosslink / kiss control

0 - standard crosslink  1,2,3 - various KISS modes

6  The Tx delay

10's of milliseconds  30 solid state, 45 for relays

7  The full duplex flag

0 - normal  1 - full duplex (we do not have any in APN)

8  The RS232 port node broadcast interval

0 - normal  600 - 10 minutes

9  The node broadcast algorithm

0 - off  2 - RS-232  1,3 - Not used

10  The beacon period

600 - 10 minutes  900 - 15 minutes  3600 - 1 hour

11  The 'connect' redirector (if user says 'Connect' only)

0 - Host  1 - BBS  2 - DXCluster

12  The 'help message' flags, case sensitivity, TALK 8 bit

0-255 add bits up as binary number (0-off, 1-on) APN:31

BIT       FUNCTION

=========================

0  y  Whether the 'please wait, trying xxxx' operates

1  y  Whether all commands appear in help for sysop

2  y  Whether the 'goodbye' message is given

3  y  Whether a welcome message is enabled ( CTEXT )

4  y  Whether nodes are shown as 'alias:callsign'

5     If set, TALK data is passed as 8 bit data rather

then clearing the most significant bit

6     If set, node aliases are case sensitive

7     If set, enables the "*** LINKED to" interface

13  The 'hash' node broadcast port control

0 - passed  1 - RS-232 OK  2 - radio OK  3 - restricted

14  Whether the node will listen for the extra aliases

0 - none  1 - monitor for extra aliases

15  Whether remote disconnect causes reconnection to the switch

0 - disconnect  1 - come back to node

16  Control over 'slime trails'

Bit 0 if set hides slime trails in nodes list

Bit 1 if set causes slime trails to be ignored

17  Control over digipeating up/down links

Bit 0 set makes node to refuse digi'd L2 uplink

Bit 1 set, node refuses to allow digi downlinks

5

MODE & PARM command syntax

Both use the same syntax, which may be either of two  types,

the  original  TheNet  1.01 syntax (  as  used  in  versions

previous to X-1J ) or an 'offset & value' type.

The original syntax was, by way of example,

PARM { [ * | new_value ] [ * | new_value ] .......... }

so to set the 10th PARM ( the L4 retries ) to 1, the  syntax

would be :

PARM * * * * * * * * * 1

The equivalent new syntax command would be :

PARM / 10 1

The '/' command signifies that what follows is the parameter

number  followed  by the new value. As for the  old  command

syntax,  the  complete  list  of  parameters  is  displayed.

Setting  the  parameters may only be done by a  Sysop.  Note

that  BOTH  command  syntaxes are supported -  you  can  use

whichever you prefer.

NODES

The entire contents of the node table routes may be obtained

by the sysop or manager by the command

NODES * *

This  will dump info on all nodes, one node per  line,  with

the following format:

Alias:call   route1   route2   route3

where  route1,  route2  and  route3  comprise  the  quality,

obsolescence  count  and  port  followed  by  the  neighbour

callsign  for each of the 3 route entries for that node.  If

any  of  the routes are in use, a chevron will be  shown  by

that route.

The  extended  command  is only for sysop use  as  it,  like

auditing and conferencing, causes the node to be a source of

a  significant  amount of data ( dumping a large  number  of

node  details can consume hundreds of buffers !!! ).  It  is

quite possible that used indiscriminately, it could cause  a

warmstart of the node. Be careful.

6

PARMS

The following parameters may be configured :

1  Maximum number of destination nodes in NODES table

typically 25, 50 or 100

2  Minimum quality for auto update                   APN: 100

3  HDLC ( radio, port 0 ) default quality            APN: 100

4  RS232 ( crosslink, port 1 ) default quality

2 node stack: 255   3 node stack: 248

5  Initial value for obsolescence counter            APN: 6

6  Minimum obsolescence for node broadcast           APN: 5

7  Auto update broadcast interval (seconds)          APN: 1800

8  Level 3 ( network ) Time To Live Initialiser      APN: 10

9  Level 4 ( transport ) timeout (seconds)           APN: 300

10  Level 4 ( transport ) retries                     APN: 2

11  Level 4 ( transport ) ack delay (seconds)         APN: 6

12  Level 4 ( transport ) busy delay (seconds)        APN: 180

13  Level 4 ( transport ) window size (frames)        APN: 4

14  Level 4 ( transport ) congestion ctrl threshold   APN: 4

15  Level 7 ( switch ) inactivity timeout (seconds)

normal: 900  may be set higher temporarily if monitored

16  Persistance for transmit delay                    APN: 64

17  Persistance slottime delay (10's of msecs)        APN: 10

18  Level 2 ( link ) T1 timeout, ie FRACK (seconds)   APN: 5

19  Level 2 ( link ) window size (packets)            APN: 2

20  Level 2 ( link ) retries                          APN: 10

21  Level 2 ( link ) T2 timeout (10's of msecs)       APN: 100

22  Level 2 ( link ) T3 timeout (10's of msecs)       APN: 18000

23  Level 2 ( link ) digipeat enable flag

0 - disable  1 - enable

24  Callsign validation flag

0 - disable  1 - enable                      APN: 0

25  Node beacon ctrl (0=off, 1=if active, 2=always)   APN: 1

The default value is 0.  Sysop must set this value to

one upon initialization of node or after a coldboot.

(Reason: if node has done a cold start on it's own

and values are reset, it will be beaconing much more

often and bring this to the attention of the Sysop.

26  CQ broadcasts enable flag

0 - disable  1 - enable                      APN: 1

7

RESET

Occasionally, the node may hang up or start 'acting funny'.

RESET [ anything-else ]

Entering the reset command alone will do a warmstart. If any

other  parameter is  entered, a  coldstart is  performed. BE

CAREFUL! This will erase any sysop entered data or commands!

Always try a warmstart first.  Keep a copy  of PARMS, MODES,

deleted  commands,  and  entered  text  so  that it  may  be

re-entered in the event a cold start is required.

ROUTES

This command  will display a list of  the routes to  neighbor

nodes that it has a direct path to.  A sysop  may lock  in  a

node  route  to a  specific  value  for  purposes  of network

control. This should  ONLY  be done in cooperation with other

sysops and/or network managers  as these values need to agree

with values set at other nodes.

ROUTE [ port# | nodecall | {+/-} | quality# ]

Plus adds the route,  minus deletes it.  The port number is 0

for the radio port and 1 for the crosslink port.  Nodecall is

the callsign and  SSID of the other node  (do not include the

alias).

SYSOP

The SYSOP command  will respond  with five numbers.  Look up

the corresponding letters and type them in order. The letter

must be the same case as what it is in the password.

The SYSOP command has been enhanced to increase the level of

security offered. One problem of the old system is that  the

password is easily visible unless the user repeats the SYSOP

command  a number of times. Even then,  correlation  between

passwords is easy, so the password needs frequent  changing.

To reduce the change period, and make it harder to discover,

the node will accept a string of characters and scan it  for

the password. Hence a response of, say, 30 or 40  characters

can  be  sent,  with a random number  of  random  characters

preceding the actual data and a random number following  it.

This does not eliminate such attacks, but if used carefully,

it makes it quite a bit harder to attack.

8

TALK

Talk  is  a  conferencing command. It  allows  a  number  of

stations to hold a  simultaneous conference. There  is  only

one conference, and stations may connect to it by connecting

to the node and issuing  the TALK command.  It may be exited

by disconnecting or issuing the command '/EXIT' at the start

of a line.  ( /EXIT may be abbreviated to /EX, and it is not

case sensitive ).

Each line sent by a user is copied to all other users in the

conference,  preceded by the callsign of the user.

Whenever  a new station enters the conference, or a  station

leaves  the conference using the '/EXIT' command, the  other

conference users get a message informing them of the  event.

These status messages are sent with the callsign of the node

rather than the user.

Finally,  when entering the TALK command, a message  may  be

sent  to all those users who are connected to the  node  but

not  otherwise doing anything. For example if  WxABC  enters

the line

TALK Hello fred, can I have a chat, type 'TALK'

Then  all other stations connected to the node,  present  in

the USER list but idle, get the message

WxABC>> Hello fred, can I have a chat, type 'TALK'

displayed on their terminal.

UI

The  UI command allows a string to be sent as a Level  2  UI

frame. The syntax is

UI dest   string_of_text_<CR>

Dest  is  a callsign like destination such as  'MAIL'.  What

will  happen is that a single UI frame will be sent  with  a

source  callsign  of  the user who entered  the  command,  a

destination callsign of dest, and the rest of the string  as

text. This command may be used by BBSes to send mail beacons

when connect to this node from another  and when digipeating

is turned off.

9

USERS

The USER command shows stations connected to the node.  When

a user connects on to another node or downlinks, an entry is

formed  showing the two links on either side of a "<-->"  or

"<..>"  symbol. In this version, the '-' characters will  be

replaced  by  another  character if one  or  other  link  is

choked. The first '-' character is associated with the  left

hand  connection ( circuit or uplink ). The second with  the

other circuit or downlink.

For a level 2 uplink or downlink, the "-" is replaced by 'C'

if the connection is choked. For Level 4 connections, the "-

"  is replaced by one of 3 characters, "R" if it  is  choked

from the other end, "L" if it is choked from this end or "C"

if it is choked from both ends!

Without a connection in progress in merely states the status

of the connection (Uplink).

Enable/Disable a command

Any command may be  enabled or disabled by the use of the '+'

or '-' modifier.

ANY_COMMAND   [ + | - | THAT_COMMANDS_PARAMETERS ]

Commands  may be caused  to be displayed or not in the prompt

screen if the '-' or '+' is followed by 'D' ( or 'd' ).

BTEXT, INFO and CTEXT Command Syntax

If  someone who is not Sysop uses the command,  the  current

settings are displayed.

If  a Sysop uses it without any additional  parameters,  the

current setting is displayed.

If  a  Sysop  enters  one of  the  commands  followed  by  a

parameter of '*', the current message is deleted.

If  a Sysop enters a string of text, that text is  added  to

the current message, followed by a newline.

It is therefore possible to build up multiple line messages.

If  you wish to start a message with a blank line,  enter  a

message  with  a  non  display  (  or  innocuous  display  )

character such as control-A. It will get entered followed by

a  newline. On most systems this will not display.  On  some

systems such as PCs running NOS, it will display as a smiley

face.

10

Resetting Configuration from Coldstart (and initial setup)

Any time the node looses  backup battery (internal lithium),

it will do a cold start, which sets it back to the condition

of initial installation. (NOTE: if X1J4 EPROM is swapped out

without resetting  Backup Battery,  memory will be retained!

You MUST reset battery or a RESET * to reset to new values.)

After a coldstart, the node operator  should enter the SYSOP

mode and do the following:

ADC - d          (removes from user prompt list)

IPROUTE - d          "      "    "     "     "

DXCLUSTER - d        "      "    "     "     "

ARP - d              "      "    "     "     "

STATS - d            "      "    "     "     "

PARM / 25 1      (resets beaconing to only when 'active')

set any other node specific differences from network

enter any INFO, BTEXT, or CTEXT messages your node has

lock routes after it has had time to initially list them

Recordkeeping

It is recommended that you keep a record of all PARMS & MODE

settings.  You can do this  easily  by  setting  your packet

program to  'Screen Capture'  and entering  the PARMS & MODE

commands  while connected to the node.  You might also enter

BTEXT,  CTEXT  and INFO,  if you have anything stored there.

Next, BBS ? and HOST ? if you have any specified.  AND don't

forget to enter ROUTES! Then make sure that the callsign and

alias  are at least written on the page  and store in a safe

place.  It is  highly recommended  to send a current copy of

this listing to  the APN coordinator.  The password  may  be

stored with this document if secure, or in a seperate place.

Record keeping is essential to good network maintenance. And

the sharing  of that information with  network adminstrators

can make all the differnece in an emergency,  if you are for

some reason absent, or unreachable.

Consultation

If you have any questions, whatsoever, about an settings, or

you are unsure of what a value should be, please contact one

of the network administrators.  They are here to help you in

anyway they can so that we can all have a smooth running and

efficient packet network.

11