ARKANSAS PACKET NETWORK X1-J QUICK REFERENCE GUIDE

==================================================================

SWITCH COMMANDS

ALIAS     [ * | NEW_ALIAS ]

BBS       [ CALLSIGN | * | ? ]

BTEXT     [ * | BEACON_MESSAGE_TEXT ]

BYE

CALIBRATE [PERIOD_VALUE_FROM_1_TO_60 [TOGGLE_VALUE_1_TO_PERIOD ] ]

CONNECT   [ CALLSIGN [ [V] DIGILIST ... ]

CQ        [ MESSAGE_FOR_CQ_PACKET ]

CTEXT     [ * | CONNECT_MESSAGE_TEXT ]

HELP

HOST      [ CALLSIGN | * | ? ]

INFO      [ SYSOP_SET_MESSAGE ]

LINKS

MHEARD    [ NUMBER_FROM_1_TO_100 ]

MODE      [ { NEWPARAM | * } { NEWPARAM | * } ..... ] (see note 2)

NODES     [* [*] | NODECALL {+|-} IDENT QUAL COUNT PORT NEIGHBOUR

[DIGIS]]

PARMS     [ { NEWPARAM | * } { NEWPARAM | * } ..... ] (see note 2)

QUIT

RESET     [ ANY_CHARACTER ]

ROUTES    [ PORT NODECALL [ DIGILIST ... ] { + | - } PATHQUALITY ]

SYSOP

TALK      [ STRING ]

UI        DEST   STRING_OF_TEXT_TO_BE_SENT_IN_UI_FRAME

USERS

------------------------------------------------------------------

Note 1 Any command may be enabled or disabled by the

use of the '+' or '-' modifier, as shown below :

ANY_COMMAND   [ + | - | THAT_COMMANDS_PARAMETERS ]

Commands may be caused to be displayed or not in

the prompt screen if the '-' or '+' is followed

by 'D' ( or 'd' )

Note 2 The commands will accept the 'old' syntax of '*

* * value...' or the new offset & value syntax of

'/ parameter_number value'

NODE INTIALIZATION AFTER COLDBOOT

C node

SYSOP

ADC - d

IPROUTE - d

DXCLUSTER - d

ARP - d

STATS - d

PARMS / 25 1

MODE / 9 2

any other node specific values

INFO *******

BTEXT ******

CTEXT ******

ROUTES  (lock in after they have been added to Routes list)

RS-232 (2 node stack): 255   RS-232 (3 node stack): 248

6m neighbor node: 224  2m neighbor node without 6m: 192

(do not lock in non-network nodes, leave them at 100)

PARMS PARAMETERS

Number Min    Max  Function

-----------------------------------------------------------

1      1    400  Maximum number of destination nodes

2      0    255  Minimum quality for auto update

3      0    255  HDLC ( radio, port 0 ) default quality

4      0    255  RS232 ( crosslink, port 1 ) default quality

5      0    255  Initial value for obsolescence counter

6      1    255  Minimum obsolescence for node broadcast

7      0  65535  Auto update broadcast interval (seconds)

8      0    255  Level 3 ( network ) Time To Live Initialiser

9      5    600  Level 4 ( transport ) timeout (seconds)

10      1    127  Level 4 ( transport ) retries

11      1     60  Level 4 ( transport ) ack delay (seconds)

12      1   1000  Level 4 ( transport ) busy delay (seconds)

13      1    127  Level 4 ( transport ) window size (frames)

14      1    127  Level 4 ( transport ) congestion ctrl threshold

15      0  65535  Level 7 ( switch ) inactivity timeout (seconds)

16      0    255  Persistance for transmit delay

17      0    127  Persistance slottime delay (10's of msecs)

18      1     15  Level 2 ( link ) T1 timeout, ie FRACK (seconds)

19      1      7  Level 2 ( link ) window size (packets)

20      0    127  Level 2 ( link ) retries

21      0   6000  Level 2 ( link ) T2 timeout (10's of msecs)

22      0  65535  Level 2 ( link ) T3 timeout (10's of msecs)

23      0      1  Level 2 ( link ) digipeat enable flag

24      0      1  Callsign validation flag

25      0      2  Node beacon ctrl (0=off, 1=if active, 2=always)

26      0      1  CQ broadcasts enable flag

==================================================================

MODE PARAMETERS

Number Min    Max  Function

-----------------------------------------------------------

1      0      1  Hardware handshake host control mode flag

2      0   3600  CWID repeat period ( seconds )

3      4     10  CWID speed ( 10's msecs per dot )

4      0      3  Nodes broadcast channel enable flags where

0=none, 1=HDLC only, 2=RS232 only, 3=Both ports

5      0      3  Crosslink protocol selection

0=crosslink, 1=KISS,

2=KISS+selective copy, 3=KISS+allcopy

6      0    255  TX keyup delay ( 10's of milliseconds )

7      0      1  Full duplex enable flag

8      0  65535  RS232 ( port 1 ) node broadcast period ( secs )

9      0      3  Node broadcast algorithm control flags

0=off, 2=RS232 port, 1 & 3 not normally used

10    600   3600  Beacon interval ( seconds )

11      0      2  Connect redirection to BBS flag

12      0    255  Help messages flags, 8 bit TALK, case & TEXNET

13      0      3  Hash node broadcast disable (one bit per port)

14      0      1  Enable extra aliases monitoring if set

15      0      1  Enable auto reconnect to node after remote dis

16      0      3  Slime trail control. Each bit controls...

Bit 0 if set hides slime trails in nodes list

Bit 1 if set causes slime trails to be ignored

17      0      3  Digipeat control. Each bit controls ...

Bit 0 set makes node to refuse digi'd L2 uplink

Bit 1 set, node refuses to allow digi downlinks

==================================================================

HELP MESSAGES FLAGS ( MODE Parameter 12 )

BIT       FUNCTION

-----------------------------------------------------------

0         If set, the 'please wait, trying xxxx' operates

1         If set, all commands appear in help for sysop

2         If set, the 'goodbye' message is given

3         If set, a welcome message is enabled ( CTEXT )

4         If set, nodes are shown as 'alias:callsign'

5         If set, TALK data is passed as 8 bit data

6         If set, node aliases are deemed to be case sensitive

7         If set, enables the TexNet "*** LINKED to" interface

=================================================================